What is Orthophotography?

Orthophotography is high quality georeferenced digital 3 band aerial photography.  Orthophotographs are scale-accurate raster images that offer a detailed bird’s-eye view of the landscape.  Any distortions caused by the earth’s surface or aircraft movement have been removed by a process of orthorectification to ensure the images are accurate. Photographs are taken in optimal weather conditions to provide the best possible images.

PRODUCT INFORMATION

• Orthophotography capture began in March 2003

• Orthophotographs are available in versions (full coverage of NI) up to 2014; they are supplied by year from 2013 (only coverage for areas flown that year)

• Each pixel represents a real-world distance of 10cm (2022 onwards), 16cm (2012-2021) for digital and 25cm (2003-2012) for film imagery

• Geo-referenced: co-ordinates can be found for any point on the map using GI software and can be used with other OSNI mapping and address data

Key Features:

- Consists of three colour bands: Red, Green and Blue

- Birds-eye view of topography of the land

- Natural and man-made features

TECHNICAL SPECIFICATION

Format – JPG, ECW

Supply – Digital

Availability – Tile

Tile Size – 2.4Km x 1.6Km

Coverage – 3971 tiles covering Northern Ireland

Update Frequency – 3 year cycle

Resolution – 10cm pixel size (2022 onwards), 16cm pixel size (2012-2021)

Maximum file size – 85MB JPG, 173MB ECW

Co-ordinate Reference System – EPSG 29903 / TM75

APPLICATIONS

- Ideal background mapping for GIS

- Planning – identify development/change in the land

- Environmental analysis/assessment

- Development and land use planning

- Watershed analysis

- Civil engineering

- Engineering design

- Asset management

- Municipal and government maps

- Right-of-way mapping

- Public and private land development

- Land-use identification

Product Enquires and Feedback

mapping.helpdesk@finance-ni.gov.uk