What is Pointer?

TECHNICAL SPECIFICATION

Format – CSV

Supply – Digital – Available as full file and updates only

Availability – LGD and Full Northern Ireland

Coverage – LGD or Full Northern Ireland

Update Frequency – Daily

Maximum file size – 378MB

Co-ordinate Reference System – ESPG 29902 / TM65

Key Features:

- UPRN

- Co-ordinate for each record

- Full address information

- Building and Address status

- Building Classification

- Administrative Boundary detail

- Townland names

- Land Parcel Fusion ID

- Building Polygon Fusion ID

Pointer is the most comprehensive and authoritative address database for Northern Ireland.  It is maintained by LPS with input from other authoritative sources, local district councils and Royal Mail.  This is the common standard address dataset with co-ordinates for every property in Northern Ireland.

PRODUCT INFORMATION

• Supplied as all NI coverage; full product available daily and update-only products available; monthly, quarterly, yearly, weekly and daily

• Individual Local Government District (LGD) datasets are supplied as full product and monthly updates

• As well as address information, Pointer records contain the building and address status and the building classification

• Townland name is added to all addresses

• A Unique Property Reference Number (UPRN) is assigned to each address

• An extension file links each Pointer record to additional administrative boundary information using the UPRN field

• A rejected file includes the rejections of incorrect addresses. Reasons for the rejection of an address includes, but is not limited to, an address no longer valid due to changes to site layout, address entered in error or duplicate address

• Can be linked with OSNI Fusion through land parcel and building IDs, known as their FusionIDs

• Each record can be linked to LPS Property Data using the UPRN field and the Transport Network using the USRN (Unique Street Reference Number)

• Pointer consists of almost 1 million points with ~910,000 as Built; Approved

• Each record contains an Irish Grid co-ordinate

• Approximately 99% match with LPS property Data

Pointer Data Structure:

| Pointer Field Name | Responsible Party |
| --- | --- |
| Organisation_Name | Royal Mail |
| Sub_Building_Name | Local Council |
| Building_Name | Local Council |
| Building_Number | Local Council |
| Primary_Thorfare | Local Council |
| Alt_Thorfare_Name1 | Local Council |
| Secondary_Thorfare | Local Council |
| Locality | Local Council |
| Townland | LPS |
| Town | Local Council |
| County | LPS |
| Postcode | Royal Mail |
| BLPU | LPS |
| Unique_Building_ID | LPS |
| UPRN | LPS |
| USRN | LPS |
| Local_Council | LPS |
| X_COR | LPS |
| Y_COR | LPS |
| Temp_Coords | LPS / Local Council |
| Building_Status | LPS / Local Council |
| Address_Status | LPS / Local Council |
| Classification | LPS |
| Creation_Date | LPS / Local Council |
| Commencement_Date | Local Council |
| Archived_Date | LPS |
| Action | LPS |
| UDPRN | Royal Mail |
| Post town | Royal Mail |
| Lp_fusionid | LPS |
| Bu_buildingid | LPS |

Product Enquires and Feedback

mapping.helpdesk@finance-ni.gov.uk